ERASMUS+    SECTOR SKILLS ALLIANCE

[PROJECT DURATION: November 2014 – October 2017]

CAD/CAM

SCHOOL MATERIAL

[WORKPACKAGE 3: Designing joint curriculum]

[OUT 3.2: Learning materials]

PREPARED BY: P12-Strojarska tehnička škola Fausta Vrančića

Author:  Snježana Korčij

Translated by: Strojarska tehnička škola Fausta Vrančića

The European Commission support for the production of this publication does not constitute endorsement of the contents which reflects the views only of the authors, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

June 2016

TABLE OF CONTENT

1 . INTRODUCTION	4

2 .  TYPES OF PROGRAMMING THE CNC MACHINE TOOLS	4

3 .  CAD/CAM	5

3.1.  CAD - Computer Aided Design	5

3.2.  CAM– Computer Aided Manufacturing	7

3.3.  Advantages and possibilities of using CAD/CAM	11

3.2.   Algorithm of CAD/CAM programming	12

4.  5-Axis processing	15

4.1. 5-axis processing in the moulds and tools industry	15

4.2. 5-axis processing of cutting tool holders	16

4.3. 5-axis processing of propellers	16

4.4. The 5-axis processing of turbine blades	17

4.5. Kinematics of CNC machines	18

5. EXAMPLE OF CAD/CAM PROGRAMMING FOR FIVE-AXIS MILLING	20

5.1. Assignment description:	20

5.2. Assignment solving procedure	22

5.2.1. Sketch analysis	22

5.2.2. 3D model development	22

5.2.3. Development of blade processing technology	27

5.2.3.1. Defining the 5-axis CNC machine for blade production	27

5.2.3.2. Defining the milling operation	29

6. TITLE, REFERENCES	37

APPENDIX 1: PART OF G CODE FOR 5-AXIS MILLING OF A TURBINE BLADE	39

APPENDIX 2: TECHNOLOGICAL DOCUMENTATION	41

LIST OF PICTURES, TABLES  AND CHARTS

Figure 1: 3D models created in two different CAD programs ready for import into CAM[13]	6

Figure 2: CAD systems[13]	7

Figure 3 CAD/CAM system[13]	8

Figure 4: CAM sytem[13]	9

Figure 5: CAM programming of CNC machine in two different CAM programs[13]	12

Figure 6: The most famous CAD/CAM systems[13]	12

Figure 7: Algorithm of  CAD/CAM programming[13]	12

Figure 8: Mould core processing[5]	15

Figure 9: Processing bevel tools[5]	16

Figure 10: Final processing of propeller blades[5]	17

Figure 11: Rough machining of a turbine blade[5]	17

Figure 12: Final processing of a turbine blade[5]	18

Figure 13: Five-axis mill[5]	19

Figure 14: A workshop blade sketch[12]	21

Figure 15: Blade profile coordinates[12]	22

Figure 16: Interpolating the previously obtained referential cross-section of the blade[12]	23

Figure 17: Extracting the body of blade[12]	24

Figure 18: Final 3D model of the blade[12]	25

Figure 19: Processed item model[13]	26

Figure 20: Model of the blade and processed item[13]	26

Figure 21: CAM module of Machining_Prismatic Machining environment[12]	27

Figure 22: Five-axis CNC machine[12]	28

Figure 23: Clamping of the workpiece on the machine[12]	29

Figure 24: Defining the processing parameters for rough milling of the blade[12]	29

Figure 25: Simulation of the blade profile rough milling on the machine[12]	31

Figure 26: Defining the processing parameters for blade’s external surfaces milling[12]	32

Figure 27: Simulation of milling the blade’s external surfaces on the machine[12]	33

Figure 28: Simulation of final finishing of blade’s external surfaces in CATIA program[12]	34

Figure 29: Simulation of face milling of blade top in CATIA  program[12]	35

Figure 30: Defining the processing parameters for face milling of the blade top[12]	35

Figure 31: Turbine propeller blade[12]	36

LIST OF APPENDIXES

Appendix 1: Part of G code for 5-axis milling of a turbine blade

Appendix 2: Technological documentation

1 . INTRODUCTION

This manual is intended for students who will attend the CAD/CAM module.

In the module classes, students will learn how to program a 5-axis CNC mill by using some of the CAD/CAM programs, how to perform the presetting of the machine and produce their first piece on the 5-axis mill.

2 .  TYPES OF PROGRAMMING THE CNC MACHINE TOOLS

CNC machine tools programming is a procedure of developing a program according to a pre-defined technology for the development of a product and it can be done manually or with the assistance of a computer.

Manual programming is the oldest and technologically the lowest level of programming the NC and CNC machines. It is implemented in the technological elaboration of processing parts of simple geometry and in case thery is a small share of CNC machines. In developing an original program of manufacturing one part (like a NC-code), a programmer-technologist guides the tool from one point to another following the the processing contour, taking into account the technological parameters: machinability of processed item, features, optimal parameters of processing, etc.

Disadvantages of manual programming are the following:

- Manual programming when there is a large number of machines and complicated parts becomes a “bottle neck” in production

- More qualified programmers are needed

- the technological procedure itself of manufacturing parts lasts longer due to the slow programming procedure which increases the product price

- there is a greater possibility for error, particularly when creating more complex parts

- Computer-aided programming means automatic programming based on selected parameters such as: dimensions of prepared item, path and selection of tools, processing regimes, etc. in specific softwares

In computer-aided programming, the NC code is generated on the basis of 3D geometry of manufactured item, the available tools and processing regimes through CAD/CAM system.

The computer-aided programming has prevailed over manual programming due to:

- Faster development of program

- Less possibility for error

- Better use of resources

# 3 .  CAD/CAM

# Cad/Cam is a technology enabling an automatic transfer from product design to its production.

# Cad/Cam represents the integration of design (CAD) and production activities (CAM) through computer systems.

# 3.1.  CAD - Computer Aided Design

CAD - Computer Aided Design is a manner of using computer programs in order to help with creating, modifying, analysing and optimizing design.

CAD programs are used in the development or improvement of product construction, from concept design to documentation production.

CAD programs represent support to the design process, in creating geometric presentation of what is being designed, then to the process of dimensioning and tolerances, managing  changes, archiving and exchange of information on parts and assemblies.

Figure 1: 3D models created in two different CAD programs ready for import into CAM[13]

CAD system consists of inter-related elements:

- User (constructor) who knows how to work on the computer and other hardware equipment, uses the CAD sofware and other necessary programs and is able to resolve an established construction problem

- Hardware consists of the computer and its equipment. Hardware needs to be adjusted to the user and it is important to ensure efficient functioning of CAD software (more RAM, faster processor, good quality monitor...) so that he can address specific construction assignments. For particular assignments, some additional specific I/O equipment is required.

- Software consists of  primarily the CAD software, but it also uses the operation system and various assisting programs. Special additional CAD software modules are used for particular purposes.

- A problem and construction assignment present an entry point into the proces of constructing and developing a product. The established assignment affects other CAD system elements. The specific problems require the constructor’s specific expert knowledge. The specific problems also require particular characteristics and/or harware and software solutions

CAD

User

Hardware

Software

Problem

Figure 2: CAD systems[13]

# 3.2.  CAM– Computer Aided Manufacturing

CAM– Computer Aided Manufacturing is an efficient use of computer technology to plan, manage and control the production processes. On the basis of CAD model, the generating of G code for processing on computer managed machines is done. Today, almost all types of processing are based on CAM technologies.

Unlike CAD, which used to help in modelling and constructing products, processes and manufacturing plants, CAM is used as program support in manufacturing, i.e. the control of CNC machines used for processing and manufacturing of processed items. Traditionally, CAM is seen as a program tool for numerical control (NC) where the CAD system helps generate two-dimensional or three-dimensional models. By using the data on model geometry obtained from the models and drawings created in the CAD system, it is possible in the CAM program to generate a tool path for different manufacturing procedures. A typical process of developing a program for CNC machine management includes manufacturing parts and lists of parts in the CAD program up to the final point and also manufacturing the layers of geometric features required for the CAM processing program.

The description of a part made in the CAD program is translated into the adjusted format, such as DXF or IGES, which is then loaded into the CAM program where it is used to make a tool path which follows the description previously made in CAD

DXF File

CAD PROGRAM

Computer

Aided  Design

Creating the geometry of all integral parts

CAM PROGRAM

Computer Aided       Manufacturing

Taking over the  geometry from  CAD and its description by G code

NC File

CNC PROGRAM

Computer Numerical Control

CNC machining

Figure 3 CAD/CAM system[13]

Similar to the CAD system, the CAM system is made up of mutually inter-connected elements.

- The CAD model represents the entry for CAM system. In defining the CAD model (in the construction phase) the manufacturing technology is already defined or the selection of possible technologies is significantly limited.

- The  hardware in the CAM system consists of, primarily the CNC machines, machining centres, flexible manufacturing systems, etc., as well as a computer. The complexity and price of CNC machines significantly exceed the complexity and price of computers, and contemporary machines usually have their own computer.

- A CAM  software can be independent or connected to a particular machine.

CAM

User

Hardware

Software

CAD model

=

Figure 4: CAM sytem[13]

The basic functions of CAM system are related to the production and technological processes planning.

They include the following:

• generating the prepared item

•generating and optimizing the tool paths

• creating and using the data bases and catalogues of cutting conditions and tools

• calculation of manufacturing time

• generating the NC programs

• simulation and visualisation of manufacturing processes

• generating the production documentation

The geometric model of the product represents the final form that needs to be achieved after manufacturing whereas the prepared item that the individual machinge element comes from has a different shape. The CAM systems have a possibility to generate the prepared item automatically on the basis of a geometric model of the product. This function is based on the so-called logic of the additional items for processing, i.e. the logic of standard dimensions of material. Using the gauge dimensions of geometric model, the Cam system generates the model of prepared item and merges it with the prodoction and tedchnological model. In some cases the prepared item takes over the size of gauge dimensions of model while keeping the standard shape of cube (for prismatic models) or of cylinder (for rotational models). The process of automatically generating the prepared item is not always possible, or is not always preferential, so the user has the possibility to intervene directly and create the prepared item.

Generating and optimizing the tool path is a CAM function usually performed in the framework of technological planning. However there are manufacturing situations requiring additional verification of tool path and possibly their correction and regeneration. These are mostly done in the manufacturing of more complex products when the tool paths are limited not just by the product configuration, but also by the configuration of machine, equipment and work environment.

Creating and using data bases and digital catalogues of machines, equipment, tools, accessories, cutting conditions and other important elements of manufacturing process is crucial for a quick and efficient modelling and production simulation. The CAM systems have special modules for this purpose.

The calculation of manufacturing time is performed automatically on the basis of other production and technological parameters taking into consideration the product size and configuration. Generally, the manufacturing time, apart from the cutting time, includes all preparatory, auxiliary and final time periods.

Prior to generating the program for CNC machine and other manufacturing equipment (robots, means of transport, etc.), and before the final process of generating the product documentation, it is necessary to conduct the simulation of individual technological processes, as well as of the entire production process. The CAM systems have very good functions for this purpose enabling the user to detect irregularities in the initially created production and technological procedure. The objective is to avoid entirely any changes before the actual production starts, or at least to minimize them as much as possible. During simulation, the CAM system visually shows all relevant information, particularly marking the places of possible problems and flaws.

The process of generating the NC program which is used to computer-manage the production equipment is entirely automatic. This is performed through special functions of CAM system, on the basis of geometric and technological model.

Generating production documentation in contemporary CAM system refers, primarily, to the creation of documentation in electronic format convenient for exchange among various participants in the product development. Since the communication of this type is carried out in the Internet environment, the generating of production documentation means automatic creation (hyper text) of document in HTML format. So, for example, for the operation of milling a prismatic machine element, a document containing all relevant data on that operation is generated.

# 3.3.  Advantages and possibilities of using CAD/CAM

Applying the CAD/CAM system provides its users with many benefits, so the question of whether to use the system or not is really not relevant. However, the prevailing issues are which optimal solutions should be applied in a particular production or development company. Below are some advantages provided by the CAM/CAD system:

- Increased production (speed) – this is one of the most important requirements for companies today. Having the right solution at the right time is a condition for progress and survival in the world of global competition.

The speed can be increased in the following way:

- Automation of routine tasks in order to increase creativity,

- Entry of standard parts from the data base,

- Fast development of prototype,

- Support changes on the construction: easy and reliable changes of construction enable, on the one hand, to remove the errors that appear in the construction process, and on

- the other hand, which is even more important, enable the possibility of creating numerious varieties and their improvements in the process of optimizing construction solutions. The easy construction changes are carried out as follows:

- There is no need for repeated drawing of all parts after each change

- The previous construction itearations are saved

- Communication, can be observed from the following aspects:

- With other teams/engineers (manufacturers, suppliers...)

- With other programs (CAD, CAM, CAE, ...)

- With marketing (realistic representation of construction)

Figure 5: CAM programming of CNC machine in two different CAM programs[13]

Regarding the market statistics, the commercial subjects dominating the CAD/CAM industry are shown in the picture.

| Manufacturer | Program |
| --- | --- |

Figure 6: The most famous CAD/CAM systems[13]

# 3.2.   Algorithm of CAD/CAM programming

The CAD/CAM  programming is conducted in several steps:

1.  Analysis of sketches and corresponding documentation

2.  Designing 3D models of processed items in one of the CAD programs

3. Defining the technology of processing, tools, cutting conditions etc., in a CAD program

4. Simulation of processing and correction in one of the CAD programs

5. Generating the NC code and technological documentation

- Selecting type of processing

- Selecting CNC machine

- Adopting the tool set

- Adopting auxiliary accessories

- Defining geometry of initial  material

- Defining additional item for processing

- Analysis of 3D model of the product created in a CAD program

- Analysis of all other information on product

Collecting information on product

Adopting processing parameters

Defining prepared item

Defining processable planes

Postprocessing

Conducting operational activities

- Automatic recognition of processable surfaces

- Defining work planes

- Generate cutting path

- Define work regimes

- Define tools

- Adjust other parameters

- Simulation of processing

- Generating NC code

- Generating documentation

Figure 7: Algorithm of  CAD/CAM programming[13]

4.  5-Axis processing

The 5-Axis processing emerged with the development of 5-axis machines with modern managing devices. Programming the 5-Axis machines is complex. That is why the programmers usually use CAD/CAM systems to program 5-Axis machines.

5-Axis processing is applied in the production of processed items with complex geometry and surfraces curved under various angles. Before 5-axis processing appeared, the problem of such processed items was solved with 3-axis processing combined with various special tools which prolonged the manufacturing time and made the product more expensive.

# 4.1. 5-axis processing in the moulds and tools industry

One of the most significant roles of processing by metal cutting is generally present in the mold manufacturing industry. The molds for the manufacturing of polymeric manufactured items can have very complicated surfaces whose geometry can only be achieved through 5-axis processing.

Figure 8: Mould core processing[5]

# 4.2. 5-axis processing of cutting tool holders

While processing holders of cutting tools for metal processing it is necessary to conduct the procedure of milling the bolsters of interchangeable plates in simultaneous 5-axis mode. In many cases it is necessary to conduct the processing at several different depths, depending on the tool type. A shorter processing time is crucial since this is a very expensive processing procedure. Shortening the processing time is only possible by ensuring a smooth tool path which is supported in the system of 5-axis management computers.

Figure 9: Processing bevel tools[5]

# 4.3. 5-axis processing of propellers

Propeller processing is one of the most complex tasks for milling process. One of the reasons for that is that a very small space for turning the tool has to be combined with high requirements for the quality of processed surface. Further on there are economic requirements to complete the production quickly in combination with the requirements for smooth passes of tool over the processed item having many processable items.

Figure 10: Final processing of propeller blades[5]

# 4.4. The 5-axis processing of turbine blades

# Rough machining of turbine blades – is a classical assignment for 5-axis processing. The 5-axis processing in this case combines the managing and programming of paths for tools with curved processable surfaces. For the processing itself, large face mills are used toghether with a simultaneous 5-axis control. In order to reduce the production time it is crucial to program valid and correct tool paths. Therefore the postprocessors are optimized in order to provide support to the managing computers in the framework of easy management of tool paths.

Figure 11: Rough machining of a turbine blade[5]

Final processing of turbine blades – there are two processing strategies for final processing of turbine blades. The first strategy, shown in Figure 9, is a strategy in which the final processing is done by ball end mill and by turning the tool at a specific angle in order to avoid collision and to optimize the conditions of surface cutting. A spiral tool path is used in this strategy in order to avoid traces on the surface caused by path stepover.

Figure 12: Final processing of a turbine blade[5]

# 4.5. Kinematics of CNC machines

The number of machine axes relates to the number of degrees of free movement or the number of possible independently controlled auxiliary parts of machine. Five degrees of freedom of movement is the minimum which enables maximum flexibility. That means that the tool and processed item in an operation can be oriented, relativelly, one towards the other at any angle. In the use of translation (T) and rotation (R) axes in the current constructions of 5-axis machines, the following axes combinations were used:

- Three translation and two rotation axes

- Two rotation and two translation axes

- One translation and four rotation axes

- Five rotation axes

The great majority of 5-axes machines falls into the category of machines with three translation and two rotation axes. The groups of machines with two translation and three rotation axes are used in some machines for the production of ship propellers. The other two groups are used in robots, usually in combination with adding some additional axes.

Figure 13: Five-axis mill[5]

5. EXAMPLE OF CAD/CAM PROGRAMMING FOR FIVE-AXIS MILLING

Below is a description of the CAD/CAM programming procedure in the CATIA V5 Program. A similar procedure is used to generate the NC code in other CAD/CAM softwares.

# 5.1. Assignment description:

According to the laws of fluid theory, the cross-section gauges of steam turbine blade are set every 40 mm, by their appearance, dimensions and cross-section aerodynamic profile.

From this initial data it is necessary to:

- Generate a blade wing by directing the model through the set cross-sections of blade in a CAD program

- Generate the NC code in a CAM program for the production of blade wing on a five-axis CNC mill.

On the basis of the construction obtained, it is necessary to define the prepared item (the raw form of the material) and production technology according to operations:

- Five-axis rough milling

- Five-axis contour milling

- Five-axis smooth (Isoparametric) milling

- Final surface finishing

It is necessary to define the tools, work regimes and work methods for each operation.

- Initiate the processing simulation and (if necessary) perform the NC code corrections

- Generate the technological documentation in a CAM program for the production of blade wind on a five-axis CNC mill.

- Produce the blade on a five-axis CNC mill.

Figure 14: A workshop blade sketch[12]

# 5.2. Assignment solving procedure

# 5.2.1. Sketch analysis

In this phase of development it is necessary to study in detail the sketch geometry, all text instructions in the sketch and the header. Special attenction needs to be paid to the surface roughness and tolerances.

# 5.2.2. 3D model development

Figure 15: Blade profile coordinates[12]

In the Part module of the CATIA V5 program, in Sketch, draw a reference cross-section of the blade as shown in Figure 15.

The blade itself, i.e. its 3D appearance is obtained by scaling and interpolating the previously obtained referential cross-section of the blade.

Figure 16: Interpolating the previously obtained referential cross-section of the blade[12]

By passing through multiple profiles of blade segments, as defined in the workshop sketch, the spatial spiral body of blade is formed.

Figure 17: Extracting the body of blade[12]

For the final 3D model, it is necessary to add the blade stand. The final appearance of 3D blade is shown in Figure 18.

Figure 18: Final 3D model of the blade[12]

Having defined the final form of the blade, also in Part the 3D model of prepared item from which the blade will be produced is added.

Figure 19: Processed item model[13]

Figure 20: Model of the blade and processed item[13]

# 5.2.3. Development of blade processing technology

The development of blade processing technology has already started by adding the prepared item in CAD model Part, and the following technology defining is done in the CAM model Machining_Prismatic Machining programa CATIA V5.

Figure 21: CAM module of Machining_Prismatic Machining environment[12]

# 5.2.3.1. Defining the 5-axis CNC machine for blade production

In order to manufacture the presented blade, it is necessary to have a 5-axis CNC machine in the plant to be able to direct the machine and to enable tool movement simultaneously in all 5 axes (X, Y, Z, A, C). The example of such a machine where the supporting items perform X, Y and Z movements, the tilting cradle enables the turning A movement and the rotation notch table enables the circular C movement, as is shown in Figure 22.

Figure 22: Five-axis CNC machine[12]

It is necessary to fix the prepared item on the table of 5-axis CNC machine, as is shown in Figure 23. The overall blade processing will be performed in one workpiece clamping.

Figure 23: Clamping of the workpiece on the machine[12]

# 5.2.3.2. Defining the milling operation

The first operation of CNC milling of the blade is Prismatic Roughing. In this process, a large quantity of material is taken off and the objective of processing is to come closer to the final form of the blade, leaving the additional item for final finish. This processing is regularly performed by 3-axis movement at a higher speed.

In the menu for defining the contour processing, the geometry of the body is determined where the processing will be done, as well as the selection of tools, processing work regime level, type of tool entry and tool exit of the machining process. In this operation the additional item remains on the contour for final finishing.

Figure 24: Defining the processing parameters for rough milling of the blade[12]

Figure 25: Simulation of the blade profile rough milling on the machine[12]

The second operation in the technology is the 5-axis milling of the blade's external surfaces. This processing is demanding and the main processing in the blade development. The tool is flexible and computer-controlled in 5 axes simultaneously with the objective to process the blade surface up to the final stage. The processing itself is performed by Isoparametric milling (so-called sweeping) - Multi Axis Isoparametric Sweeping Machining.

Figure 26: Defining the processing parameters for blade’s external surfaces milling[12]

Figure 27: Simulation of milling the blade’s external surfaces on the machine[12]

The third operation in the technology is the 5-axis finishing by blade surface sweeping. This process is the final one in the blade production. This is a repeated previous processing but this time the work regimes are different and slower, the tools are of smaller diameter, with increased movements to finish and polish the curves that remained from the previous processing on the blade’s surface.

Figure 28: Simulation of final finishing of blade’s external surfaces in CATIA program[12]

What remains is to level out the top of the blade using a face mill.

Figure 29: Simulation of face milling of blade top in CATIA  program[12]

Figure 30: Defining the processing parameters for face milling of the blade top[12]

With face milling operation the blade processing is completed and the result is the processed blade on the stand as shows Figure 31.

Figure 31: Turbine propeller blade[12]

After having defined the technology and completed the simulation and correction, G code is generated (Annex 1) together with technological documentation (Annex 2).

6. TITLE, REFERENCES

[1] Bošnjaković, Mladen; Stojić, Antun: Programiranje CNC strojeva. Slavonski Brod; Veleuĉilište u Slavonskom Brodu, 2011., 386 str.

[2] Bozdoc, Marian: Marian Bozdoc's history of CAD. Auckland, Novi Zeland URL : http://www.mbdesign.net/mbinfo/CAD-History.htm (21.06.2016)

[3] The best CAD software history on the web, 2004, CAD software – history of CAD CAM, URL: http://www.cadazz.com/cad-software-history.htm (21.06.2016)

[4] Novak-Marcincin, Jozef; Petik, Anton : Computer Aided Manufacturing-Inesparable part of CAD/CAM/CAE systems. Košice, Technical University of Košice, 2002

[5] FSB, Zagreb https://www.fsb.unizg.hr/atlantis/upload/newsboard/29_06_2011__15223_Franic_spec_21.05.11.pdf (22.06.2016)

[6] PTC, The Industry’s Leading 3D CAD Software, 2016, Creo Parametric URL:http://www.ptc.com/cad/creo (22.06.2016)

[7] Dassault Systemes, Company History, 2016, URL: http://www.solidworks.com/sw/183_ENU_HTML.htm (23.06.2016)

[8] Haas Automation, Inc – CNC Machine Tools, About Haas Automation, 2016, HAAS HISTORY URL:http://int.haascnc.com/about_history.asp?intLanguageCode=1033 (23.06.2016)

[9] Haas Automation ®, Inc, Haas VM Series, Prosinac 2015, The Vertical Mold Making Machine URL:http://www.haascnc.com/mt_spec1.asp?id=VM&webID=MOLD_MACHINE_VMC#gsc.tab=0 (23.06.2016)

[10] Haas Automation ®, Inc, CNC Rotaries & Indexers: Rotary Tables, 2016. URL:http://www.haascnc.com/mt_spec1.asp?id=HRT210&webID=ROTARY_TABLE_ROTARY#gsc.tab=0 (21.06.2016)

[11] Haas Automation Inc.: Korisnički priručnik za okomitu glodalicu, 2014 URL:http://diy.haascnc.com/sites/default/files/Locked/Manuals/Operator/2014/Mill/Translated/Mill_Operators_Manual_96-HR8200_Rev_A_Croatian_January_2014.pdf (21.06.2016)

[12] Marijan Abramović , STŠ Fausta Vrančića

[13] STŠ Fausta Vrančića, URL:http://www.stsfv.eu/(21.06.2016)

# APPENDIX 1: PART OF G CODE FOR 5-AXIS MILLING OF A TURBINE BLADE

N10 ;===========================================================

N11 ;==   Copyright Cenit AG Systemhaus             (mr) 2004 ==

N12 ;==                                                       ==

N13 ;==   For demonstration use only:                         ==

N14 ;==   cPost-Postprocessor for Zimmermann FZ30             ==

N15 ;==           with controller SINUMERIK 840 D             ==

N16 ;===========================================================

N17 ; PRGNR : Manufacturing Program.1

N18 G0 G90 G40

N19 TRAORI

N20 ;=============== TOOL CHANGE =================

N21 T1 M06     ; T1 End Mill D 10

N22 ;MACHINE OPERATION = Prismatic roughing

N23 ;OPERATION NAME    = Prismatic roughing.1

N24 ;TOOL ASSEMBLY     = T1 End Mill D 10

N25 ;(---------------------------------------------)

N26 MSG(" MO : Prismatic roughing.1 ")

N27 ;(---------------------------------------------)

N28 TRAORI(1)

N29 D1

N30 G00 X43.411 Y-28.1 Z307.073 A0 C0 S70 M3

N31 G01 Z297.073 F300

N32 Y-20

N33 X-10 F1000

N34 Y30

N35 X-1.461

N36 X-1.458 Y27.625

N37 X-1.455 Y25.25

N38 X11.416

N39 X1.202 Y18.19

N40 X-4.315 Y14.476

N41 X-4.435 Y14.39

N42 X-4.669 Y14.212

N43 X-4.783 Y14.119

N44 X-5.25 Y13.718

N45 Y25.179

N46 X-5.732 Y25.732

N47 X-5.179 Y25.25

N48 X-1.455

N49 X-1.453 Y23.35

# APPENDIX 2: TECHNOLOGICAL DOCUMENTATION

Machine : ZIMMERMANN_FZ30

Type : MfgGenericMillMachine

Numerical control attributes

| Axial/Radial movement | false |
| --- | --- |
| MFG_SPLIT_CIRCLE_STRAT | No split |
| Helical Interpolation | false |
| Rapid feedrate | 60000mm_mn |
| Post Processor words table | CPOST_MILL.pptable |
| Home point strategy | FROM |
| NC data format | Axis (X,Y,Z,I,J,K) |
| NC data type | ISO |
| Max machining feedrate | 100000mm_mn |
| Min interpol. radius | 0,01mm |
| Min discretization step | 1e-004mm |
| Min discretization angle | 0,1deg |
| 2D circular interpol. | true |
| 3D circular interpol. | true |
| 3D Nurbs interpolation | false |
| 3D linear interpol. | true |
| Max interpol. radius | 5000mm |
| Controller Emulator | ZIMMERMANN_FZ30_S840D.ce |

| Spindle attributes Home point X; 0mm. Home point Y; 0mm. Home point Z; 1469mm. Orientation K; 1. Orientation J; 0. Orientation I; 0. | Tool change attributes Tool change point X; 0mm. Tool change point Y; 0mm. Tool change point Z; 100mm. Radius compensation; false. |
| --- | --- |

| SHOP FLOOR DOCUMENTATION Krilo.CATProcess |
| --- |
|  | Part operation : Part Operation.1 |

Manufacturing Program.1

| Tool Change.1 Type : Tool Change |  | Tool : T1 End Mill D 10 type : End Mill |
| --- | --- | --- |
| Prismatic roughing.1 Type : Prismatic roughing |  | Tool : T1 End Mill D 10 type : End Mill |
| Tool Change.2 Type : Tool Change |  | Tool : T2 End Mill D 10 type : End Mill |
| Isoparametric Machining.1 Type : Isoparametric Machining |  | Tool : T2 End Mill D 10 type : End Mill |
| Tool Change.4 Type : Tool Change |  | Tool : T3 Face Mill D30 type : Face Mill |
| Facing.3 Type : Facing |  | Tool : T3 Face Mill D30 type : Face Mill |

| Program cutting time | : | 1h 4' 1'' |
| --- | --- | --- |
| Program total time | : | 1h 10' 49'' |

| SHOP FLOOR DOCUMENTATION Krilo.CATProcess |
| --- |
|  | Part operation; :; Part Operation.1. Program; :; Manufacturing Program.1. |

Prismatic roughing.1

Prismatic roughing

Strategy

| IPMGenMode | None |
| --- | --- |
| Machining quality | Rough |
| Nurbs Output | false |
| Unused | 1mm |
| Part contouring | true |
| Drilling tool length | 100mm |
| Unused | false |
| Unused | 10deg |
| Unused | M3xTrochoidParamModeDist |
| Direction of cut | Climb |
| Machining mode | Outer part |
| Offset on bottom plane | 0mm |
| Unused | 100 |
| Semi finishing thickness on bottom | 0mm |
| Unused | false |
| Unused | 1mm |
| Unused | 10deg |
| Unused | true |
| Engagement mode | From outside |
| Bottom thickness | 1mm |
| Unused | 0mm |
| Unused | false |
| Machining tolerance | 0,1mm |
| Drilling tool angle | 30deg |
| High speed machining | false |
| Tool path style | Helical |
| Unused | 45deg |
| Ramping angle | 15deg |
| Fully engaged tool management | None |
| Rework threshold | 0,3mm |
| Helix diameter (% O) | 70 |
| Maximum full material cut depth | 2,5mm |
| Machine only ordered areas | false |
| Approach distance | 10mm |
| Maximum depth of cut | 5mm |
| Drilling tool diameter | 25mm |
| M3xIgnoreHoleInt | 2 |
| M3xIgnoreHoleLength | 10mm |
| Pass overlap (diameter ratio) | 50 |
| Unused | 120deg |
| Drilling safety distance | 0mm |
| Pass overlap (length) | 5mm |
| Unused | 75 |
| Overshoot | true |
| Unused | true |
| Unused | -1mm |
| Radial safety distance | 3mm |
| M3xIgnoreHole | false |
| Minimum trochoid radius | 1mm |
| Unused | true |
| Unused | true |
| Engagement mode | Ramping |
| Part contouring ratio | 10 |
| Unused | true |
| Unused | 75 |
| Unused | 30mm |
| Offset on top plane | 0mm |
| Horizontal areas detection | true |
| Unused | false |
| Truncated transition paths | false |
| Unused | 1mm |
| Corner radius | 1mm |
| Tolerance | 0,3mm |
| Offset | 0mm |
| Unused | 0,5deg |
| Pass overlap mode | Overlap ratio |
| Unused | 0mm |
| Unused | 0mm |
| Helical movement | Both |
| Unused | 25 |
| Axial safety distance | 10mm |
| Stay on bottom | true |
| Unused | 45deg |
| M3xEngageFromExternalZone | false |
| M3xCircularApproach | false |
| M3xRadiusCircularApproach | 2mm |
| M3xAngleCircularApproach | 180mm |
| M3xHardRampOffset | 0mm |
| Cutting mode | Climb |
| M3xTrochoidSurface | 70 |
| M3xTrochoidNonMachiningRadius | 100 |
| M3xTrochoidStep | 0,5mm |
| M3xTrochoidMode | M3xTrochoidAuto |
| M3xRadialFirst | false |
| MfgWorkType | MfgCartesian |
| M3xFMTrochoidMaxEngagement | 0,5mm |
| Circular Interpo. | false |

| Feedrate Feedrate unit; Linear. Spindle output; true. Automatic compute from tooling Feeds and Speeds; true. Spindle unit; Angular. Automatic compute from tooling Feeds and Speeds; true. MFG_FEED_SELECT_TRANSITION; false. MFG_FEED_TYPE_TRANSITION; Machining. Maximum radius; 1mm. Distance after corner; 1mm. Minimum angle; 45deg. Reduction rate; 80. Feedrate reduction in corners; false. SlowdownRate; 100. Distance before corner; 1mm. Machining feedrate; 1000mm_mn. Machining spindle; 70turn_mn. MFG_LOCAL_FEED_TRANSITION_VALUE; 5000mm_mn. Approach feedrate; 300mm_mn. Retract feedrate; 1000mm_mn. | Machining Time Cutting time; 42' 20''. Total time; 47' 59'. |
| --- | --- |
| SHOP FLOOR DOCUMENTATION Krilo.CATProcess |
|  | Part operation; :; Part Operation.1. Program; :; Manufacturing Program.1. |

Isoparametric Machining.1

| Isoparametric Machining |  |
| --- | --- |

Strategy

| IPMGenMode | None |
| --- | --- |
| Machining quality | Rough |
| Nurbs Output | false |
| Skip path | None |
| Compensation output | 3D Contact (G29/CAT3Dxx) |
| Offset on tilting guide | 0mm |
| Thru a guide mode | Normal to the path |
| Check allowed gouging | 0mm |
| Part accuracy | 0,05mm |
| End extension | 0mm |
| Machining tolerance | 0,05mm |
| Radial strategy | Scallop height |
| Number of paths | 1 |
| Extend tilting guide | true |
| Side of tilting guide offset | Left |
| Covering mode | false |
| Check accuracy | 0,1mm |
| Unused | true |
| Unused | true |
| Collision on part | false |
| Max discretization step | 10000mm |
| To / From | To |
| Tool axis guidance | Fixed lead and tilt |
| Tilt angle | 25deg |
| Lead angle | 0deg |
| Mfg5AxisShortMacroFlag | false |
| Unused | 0mm |
| Unused | 0mm |
| Max distance between paths | 5mm |
| Tool path style | Zig zag |
| Max discretization angle | 63deg |
| Allowed lead | 30deg |
| Guidance | Lead angle |
| Min lead angle | -30deg |
| Allowed tilt | 30deg |
| Scallop height | 0,05mm |
| Collision checking mode | Active Part |
| Max lead angle | 30deg |
| Start extension | 0mm |
| Part allowed gouging | 0mm |
| Minimum heel distance | 1mm |
| Lead angle | 0deg |

| Feedrate Feedrate unit; Linear. Spindle output; true. Automatic compute from tooling Feeds and Speeds; true. Spindle unit; Angular. Automatic compute from tooling Feeds and Speeds; true. MFG_FEED_SELECT_TRANSITION; false. MFG_FEED_TYPE_TRANSITION; Machining. Machining feedrate; 1000mm_mn. Machining spindle; 70turn_mn. MFG_LOCAL_FEED_TRANSITION_VALUE; 5000mm_mn. Approach feedrate; 300mm_mn. Retract feedrate; 1000mm_mn. | Machining Time Cutting time; 21' 28''. Total time; 22' 8''. |
| --- | --- |