Machine Vision assessment paper

Name and surname: ____________________________	 Date:  _____________________

| Ʃ 100 points |  |
| --- | --- |

Points:

Written test

| Ʃ 10 points |  |
| --- | --- |

Learning outcome 1 (execusion):  Explain the relationship and impact of parameters on the digital image (10 points)

- Camera has lens Focal length 8 mm and observing part of the object is 12 mm x 12 mm. At which the working distance we have to mounting the camera?

| 1 point |  |
| --- | --- |

Source: Cognex installation manual

| 1 point |  |
| --- | --- |

- Distance between the camera and subject is 220 mm, lens Focal length is 6 mm. Specify the horizontal Field of View (mm).

- Camera has resolution 800 x 600 mm and working distance 400 mm. Observing object has hole with diameter 4 mm. How many pixels are on the length of diameter?

| 1 point |  |
| --- | --- |

| 1 point |  |
| --- | --- |

- What is the minimum number of frame rate if the maximum time of image processing is 50 ms?

- Name at least two software tools for the part location?

| 1 point |  |
| --- | --- |

- Name at least two software tools for the geometry recognition?

| 1 point |  |
| --- | --- |

| 1 point |  |
| --- | --- |

- Name at least two software tools for the Feature comparison.

Oral test

| 1 point |  |
| --- | --- |

- Explain your choice of software tools for coins value recognition.

| 1 point |  |
| --- | --- |

- Explain your choice of software tools for damage coins recognition.

| 1 point |  |
| --- | --- |

- Explain the system settings you choice to identify unsuitable currency (kuna).

Practical exercise

Using Machine Vision as a tool for identifying coins. The machine vision system will identify  coins on the basis of their characteristics such as diameter, edge width, currency (€, kuna) … Each candidate will have to define the basic feature of the object, camera settings, lens and recognition of a foreign object (unsuitable coin, token).

| Ʃ20 points |  |
| --- | --- |

Learning outcome 2 (execusion):  Prepare imaging environment and set lighting

- The candidate suggested or chose the appropriate lens.

| 2 points |  |
| --- | --- |

- The candidate set the camera properly according to the camera-to-object distance.

| 2 points |  |
| --- | --- |

- The candidate set suitable lighting settings according to the measured object.

| 2 points |  |
| --- | --- |

- The candidate selected a suitable feature of the object for ligthing the object.

| 2 points |  |
| --- | --- |

- The system recognizes the value of the coin according to the currency.

| 2 points |  |
| --- | --- |

- The candidate selected a suitable feature of the object to recognize the value of the coin.

| 2 points |  |
| --- | --- |

- The system recognizes a foreign object (foreign currency, token ... ).

| 2 points |  |
| --- | --- |

| 2 points |  |
| --- | --- |

- The candidate selected a suitable feature of the object for recognizing a foreign object.

| 2 points |  |
| --- | --- |

- The system recognizes a damaged coin edge.

- The candidate selected a suitable feature of the object for recognizing a damaged coin edge.

| 2 points |  |
| --- | --- |

Learning outcome 3 (planning):  Choose an optimum camera and lens

| 5 points |  |
| --- | --- |

The candidate explains the choice of lens and camera.

Learning outcome 4 (execusion):  Connect camera with PLC and create inspection program

| Ʃ20 points |  |
| --- | --- |

| 10 points |  |
| --- | --- |

- The connection between PLC and camera is working.

The pre-prepared programme on the PLS responds appropriately.

| 10 points |  |
| --- | --- |

Learning outcome 4 (documentation):  Connect camera with PLC and create inspection program

| 5 points |  |
| --- | --- |

| Picture of suitable object | Picture of unsuitable object |
| --- | --- |

Based on taken photos, the candidate explains why the system recognized a particular object as suitable/unsuitable.

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Learning outcome 5 (execusion and documentation): Measure and take a test on the operation of the machine

| 20 points |  |
| --- | --- |

Test system operation on a sample of 50 objects/coins. Explain the results.

________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Learning outcome 4 (planning):  Connect camera with PLC and create inspection program

| 5 points |  |
| --- | --- |

The candidate planned the software on the camera properly.

Learning outcome 5 (presentation): Measure and take a test on the operation of the machine

| 15 points |  |
| --- | --- |

The candidate demonstrates and discusses the operation of the system.