COMMISSION DECISION

of XXX

on the safety requirements to be met by European standards for certain children's seats pursuant to Directive 2001/95/EC of the European Parliament and of the Council

(Text with EEA relevance)

THE EUROPEAN COMMISSION,

Having regard to the Treaty on the Functioning of the European Union,

Having regard to Directive 2001/95/EC of the European Parliament and of the Council of 3 December 2001 on general product safety, and in particular Article 4(1)(a) thereof,

Whereas:

- European standardisation helps to boost the competitiveness of enterprises by facilitating in particular the free movement of goods. Standards produce significant positive economic effects, for example by promoting economic interpenetration on the internal market and encouraging the development of new and improved products or markets and improved supply conditions. Standards thus normally increase competition and lower output and sales costs, benefiting economies as a whole.

- European standards play a very important role within the internal market, mainly through the presumption of conformity of products to be made available on the market with the essential requirements of those products laid down in European Union legislation.

- The Directive 2001/95/EC distinguishes between two types of European standards, namely those the references of which have been published by the Commission in the Official Journal ‘in accordance with Article 4’ (Article 3(2) of the Directive 2001/95/EC), and the ‘other’ European standards (Article 3(3)(a) of the Directive 2001/95/EC).

- The European standards, the references of which have been published by the Commission in the Official Journal in accordance with Article 4 of Directive 2001/95/EC, provide that products which conform to voluntary national standards transposing such European standards benefit from a presumption of safety as far as the risks and risk categories covered by those national standards are concerned.

- For the ‘other’ European standards (being those the references of which have not been published in the Official Journal), conformity with national standards transposing them does not give rise to a presumption of safety. Under Article 3(3)(a) of the Directive 2001/95/EC, conformity with such standards constitutes merely one element among other elements to be taken into account when assessing compliance with the general safety requirement laid down in Article 3(1) of the Directive 2001/95/EC.

- Article 4(1) and Article 4(2) of Directive 2001/95/EC set out the procedure for drawing up European standards which consists of four steps: first, the Commission, following a comitology procedure, determines the requirements intended to ensure that products which conform to these standards satisfy the general safety requirement; second, on the basis of these requirements, the Commission, following the procedure laid down in Directive 98/34/EC of the European Parliament and of the Council of 22 June 1998, laying down a procedure for the provision of information in the field of technical standards and regulations and of rules on Information Society services, calls upon the European standardisation bodies to draw up standards which satisfy these requirements; third, on the basis of this mandate, the European standardisation bodies, following the principles contained in the general guidelines for cooperation between the Commission and those bodies, adopt the standards; and fourth, the Commission publishes in the Official Journal the references of these European standards.

- No European standards exist for children's chairs or for chair mounted seats. EN 14988-1:2006 ‘Children's high chairs - Part 1: Safety requirements’ and EN 14988-2:2006 ‘Children's high chairs - Part 2: Test methods’ do not set specific requirements to address the hazard of falling when children try to stand up, lose balance and fall down from the chair, which may result in head injuries including a fractured skull, contusions and lacerations. Furthermore, the hazard of strangulation by ribbons and cords is not taken into consideration.

- EN 1272:1998 ‘Child care articles - Table mounted chairs - Safety requirements and test methods’ is not referenced in the Official Journal and national standards transposing this European standard do not give rise to a presumption of safety.

- The measures provided for in this Decision are in accordance with the opinion of the Committee set up under Article 15 of Directive 2001/95/EC. Neither the European Parliament nor the Council has opposed them.

HAS ADOPTED THIS DECISION:

Article 1

Definition

For the purposes of this Decision, the following definitions shall apply:

‘Chair mounted seat’ means a product intended to be fixed on an adult chair to raise the sitting position of a child that is able to sit unaided.

‘Children’s chair’ means a seat intended for seating one child with a size adjusted to the child's age and intended to be placed on the floor level.

‘Children’s high chair’ means a free standing chair that elevates the child to approximately dining table height, intended for feeding a child who is able to sit up unaided.

‘Table mounted chair’ means a chair normally used for children that is intended to be attached to a table or other horizontal surface.

Article 2

Safety requirements

The specific safety requirements for children's seats referred to in Article 1 to be met by European standards pursuant to Article 4 of Directive 2001/95/EC are set out in the Annex to this Decision.

Article 3

Entry into force

This Decision shall enter into force on the twentieth day following that of its publication in the Official Journal of the European Union.

Done at Brussels, […]

- For the Commission

José Manuel BARROSO
	The President

ANNEX

General safety requirements applicable to all products referred in Article 1

Products shall not jeopardize the safety or health of children and carers when they are used as intended or in a foreseeable way, bearing in mind the behaviour of children.

Labels on products or on their packaging and the instructions for use which accompany them shall draw the attention of users to the inherent hazards and risks of injuries involved in using the products and to ways of avoiding them.

Chemical requirements

All children's seats shall comply with relevant EU legislation.

Fire and thermal properties

No children's seat shall constitute a dangerous flammable element in the child's environment. Therefore, children's seats shall be composed of materials that shall not burn if directly exposed to a flame or spark.

Packaging

Bags made of flexible plastics which are used for packaging and which have an opening perimeter that is larger than child's head circumference, shall note pose a risk of suffocation. Their means of closing shall not be a drawstring or cord.

The packaging in which the products are contained shall not present a risk of suffocation caused by an external airway obstruction of the mouth and nose.

These bags shall be conspicuously marked with a warning conveying the following (or equivalent) message: ‘WARNING! Keep packaging away from children to avoid suffocation’.

Identification of manufacturer and importer

Manufacturers shall indicate their name, registered trade name or registered trade mark and the address at which they can be contacted on the product or, where that is not possible, on its packaging or in a document accompanying the product. The address must indicate a single point at which the manufacturer can be contacted.

Where applicable, importers shall indicate their name, registered trade name or registered trade mark and the address at which they can be contacted on the product or, where that is not possible, on its packaging or in a document accompanying the product.

Specific safety requirements applicable to products referred in Article 1

# Chair mounted seats

## Risks

The main risks associated with chair mounted seats are:

- shearing and crushing in folding products,

- entrapments in gaps and openings,

- falls: in particular, an active child in a chair mounted seat may move it to the edge causing the chair mounted seat to fall off the chair or if the child is not restrained properly or left unrestrained the child may fall off the chair mounted seat or may fall through the gaps of the chair mounted seat.

## Scope

These safety requirements are applicable to chair mounted seats intended for children up to an age of 3 years or a maximum weight of 15 kg. They do not apply to cushions, pads and any products only aimed at restraining the child on a chair without raising the child's sitting position.

## Safety requirements

Entrapment hazards from gaps and openings

Chair mounted seats shall be designed and manufactured in order to prevent entrapment of any of child's body parts.

Hazards due to height adjustment of the product

Chair mounted seats in which the height of the sitting area can be adjusted shall have locking mechanism(s) to maintain the chair mounted seat in its position of normal use. Unintentional release of locking mechanism(s) shall be prevented.

Hazards due to moving parts

After the chair mounted seat is set up for normal use there shall be no accessible compression or shear points as the result of the movement of the chair mounted seat or any part of it, the shifting of body weight by the child using the chair mounted seat or the application of an external force (either by another child or, unintentionally, by the carer, or by a powered mechanism).

Chair mounted seats designed to fold shall have a folding mechanism incapable of being operated by a child or by inadvertent action made by the carers. It shall not be possible to set up chair mounted seat for normal use without the locking mechanism being activated.

Falling hazards

The chair mounted seat shall be fitted with a restraint system that is adjustable to the size of the child and shall comprise of at least a waist and a crotch restraint. It shall not be possible to use the restraint system without the crotch restraint being used.

The restraint system, straps, anchoring points and fastening system shall not break, become loose or tear away from their support under the internal and external forces capable of being applied by a child. The restrain system shall remain unaltered when manipulated by the child.

Entanglement hazards

Cords, ribbons and similar parts, excluding the child restraint system and chair attachment system, that are accessible from inside the chair mounted seat shall have a maximum free length which prevents a formation of a hazardous loop around a child's neck.

Monofilament threads shall not be used as cords, ribbons and similar parts, loops or as sewing threads.

Choking hazard

In order to prevent choking hazards, there shall not be any small parts, whether intended to be removed without the use of a tool or not, which could become detached by forces that a child can apply and which could enter entirely into a child’s mouth.

In order to prevent choking hazards, stuffing materials that present choking hazards shall not become accessible when submitted to the forces that a child can apply.

Ingestion hazards

To avoid the hazard of ingestion, separate or small parts which are detachable by forces that a child can apply and which can pass in the oesophagus shall be avoided.

Suffocation hazards

There shall not be any plastic decals which could be gripped by a child and detached by forces that a child can apply. There shall not be any impermeable sheeting which could cover both the mouth and the nose of the child and present suffocation hazards.

Hazardous edges, corners and protruding parts

All accessible edges, corners and protruding parts on the chair mounted seat shall be rounded and free from burrs.

Structural integrity

The chair mounted seat shall not collapse or show any sign of damage or permanent deformation which can impair its safety and normal function and any mechanism(s) allowing the adjustment of the height of the chair mounted seat shall not allow any change from the original height position due to mechanical stresses to which they are subjected during normal use.

Chair attachment system

The chair attachment system shall be designed to fix the chair mounted seat both to the chair backrest and to the chair seat surface.

The chair attachment system, straps, anchoring points and fastening system shall not break, become loose or tear away from their support due to mechanical stresses to which they are subjected during normal use.

Hazards due to inadequate size

The adequate dimensions of the seat and of the backrest of the chairs for which the product is intended shall be specified in the product information.

## Safety information, user manual and marking

General

The safety information shall be marked on the product or included in the instructions to the user.

The safety information shall be given in the language(s) of the country in which the chair mounted seat is offered for retail sale. All markings shall remain legible and any label used for the marking shall not detach easily.

Safety information

Safety information shall be provided, in a clear and conspicuous way. The chair mounted seat shall have the safety information visible after attachment to the adult chair and before the child is put into the chair mounted seat. This information shall have a heading ‘WARNING!’ and shall convey at least the following (or equivalent) messages:

- ‘Never leave the child unattended.’

- ‘Always use the child restraint and chair attachment systems.’

- ‘Always check the security and the stability of the chair mounted seat before use.’

The safety information sentences can be replaced by suitable pictograms.

Purchase information

Purchase information shall be available at the point of sale and shall contain at least the following (or equivalent) information:

- ‘This product is intended for children up to 3 years old, weighing a maximum of 15 kg, who are able to sit up unaided’

- the adequate dimensions of the adult’s chair seat and backrest, as relevant.

This information may be given by means of a pictogram.

User manual

The chair mounted seat shall come with a user manual. It shall comprise:

- the following (or equivalent) statement ‘IMPORTANT! KEEP FOR FUTURE REFERENCE.’

- the instructions of the correct and safe assembly and use of the chair mounted seat;

The warnings included in the user manual shall be labelled with the heading ‘WARNING!’ and shall convey at least the following (or equivalent) messages:

- ‘Never leave the child unattended.’

- ‘Always use the child restraint and chair attachment systems.’

- ‘Ensure the chair attachment system is correctly fitted and adjusted before use.’

- ‘Always check the security and the stability of the chair mounted seat before use.’

Additionally, the user manual shall provide the following information:

- the statement (or equivalent)‘This product is intended for children up to 3 years old, weighing a maximum of 15 kg, who are able to sit up unaided’.

- on the adequate dimensions of the adult’s chair seat and backrest, as relevant.

- A statement not to use the chair mounted seat if any part is broken, torn or missing.

- A statement not to use accessories or replacement parts other than those approved by the manufacturer.

- Cleaning and maintenance recommendations, if applicable.

# Children’s chairs

## Risks

The main risks associated with children’s chairs are:

- Entrapment of fingers and hands in foldable children’s chairs due to the failure of the locking device of the folding mechanism.

- Choking hazards due to small parts fitted at the end of the legs of the chairs that could be detached and entered into the child's mouth.

- Falling hazard due to the seat base of a chair which could detach from the leg base and could cause the child to fall to the floor.

## Scope

These safety requirements apply to children's chairs which are intended for children from two to five years old. These include stools, chairs (assembly parts include: legs, seat base and backrest) and armchairs (assembly parts include: seat base, backrest and arms) for domestic indoor and outdoor use. Rocking chairs and foldable chairs are also included within the scope.

## Safety requirements

Entrapment hazards from gaps and openings

Children’s chairs shall be designed and manufactured in order to prevent entrapment of limbs, feet and hands and as far as possible entrapment of fingers in gaps and openings.

Foldable children’s chairs shall be designed and manufactured in order to prevent entrapment of fingers.

Hazards due to moving parts

Children's chairs designed to fold shall have a folding mechanism incapable of being operated by a child or by inadvertent action on the part of the carers. It shall not be possible to erect the product without the locking mechanism being activated.

Falling hazards

To prevent the chair and the child falling over, the children’s chair shall have sufficient stability when a child leans in any direction, tries to stand up or pushes the chair.

Choking hazard

In order to prevent choking hazards, there shall not be small parts which are detachable by forces that a child can apply and which can fit completely into a child’s mouth. In order to prevent choking hazards, stuffing materials that present choking hazards shall not become accessible when submitted to the forces that a child can apply.

Suffocation hazards

There shall not be plastic decals which are detachable by forces that a child can apply and other impermeable sheeting of the children’s chairs which can cover both the mouth and the nose and present suffocation hazards.

Ingestion hazards

To avoid the hazard of ingestion, separate or small parts which are detachable by forces that a child can apply and which can pass in the oesophagus shall be avoided.

Hazardous edges

Children's chairs shall not have sharp edges and points. Accessible edges and corners shall be rounded and chamfered. There shall not be pointed or protruding surfaces that can create a puncture hazard.

Structural integrity

Children’s chair and components such as seat base, backrest, legs, shall withstand mechanical stresses to which they are subjected during use, for the lifetime of the product.

## Safety information

Warnings and instructions for use shall indicate to the parents or carers that children's chairs located under a window can be used as a step by the child and cause the child to fall out of the window.

# Children’s high chairs

## Risks

The main risks associated with children’s high chairs are:

- shearing and crushing in folding products,

- entrapments in gaps and openings,

- falls: in particular, an active child if not restrained properly or left unrestrained may fall off the high chair or fall through the gaps of the high chair.

## Scope

These safety requirements are applicable to high chairs intended for children from 6 months to 36 months of age.

If parts of the high chair are designed to be removable (e.g. a tray or a footrest), these safety requirements apply to the high chair with and without these part(s).

## Safety requirements

General

Connecting screws for direct fastening shall not be used for the assembly of any component that is designed to be removed or loosened when dismantling the high chair for the purpose of transportation or storage.

Exposed edges and protruding parts shall be rounded or chamfered and free from burrs and sharp edges.

Falling hazards

In order to avoid injuries due to the child standing up and falling out of the chair, the high chair shall be designed so that the restraint system of the chair prevents the movement of the child from standing up in the high chair and from slipping out of it. All the possible movements of the child in the high chair shall be considered. The restraint system should not be easily opened by a child in order to stay easily opened by the carers.

Entanglement hazards

There shall be no cords, ribbons and similar parts present (with the exception of the restraint system) on the high chairs that could create entanglement hazards.

Entrapment hazards from gaps and openings

Children's high chairs shall be designed and manufactured in order to prevent entrapment of any of child's body part.

Hazards due to moving parts

To avoid the risk of shearing and crushing, shear and compression points shall be avoided. If shear and compression points cannot be eliminated for functional reasons, then appropriate solutions to ensure safety should be provided.

Any part of the product which can fold or be detached shall be locked to avoid release by the child using the product, by another child or by an unintentional action by an adult.

Locking mechanisms for folding high chairs

Locking mechanisms are required to prevent a high chair from folding whilst a child is in the high chair and also when a child is being put in and taken out of the high chair.

To avoid the hazard due to incomplete deployment, either the weight of the child using the product shall act to prevent the folding or at least one locking mechanism shall engage automatically when the product is deployed for use.

Unintentional release or operation by a child of the locking mechanisms shall be prevented.

The high chair shall not collapse and the locking mechanism shall remain engaged when they are subjected to mechanical stresses reasonably foreseeable during the normal use of the high chair.

Any locking mechanisms shall continue to function properly when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

Choking hazards

No part that can be detached by the force applied by a child shall enter entirely into a child’s mouth. Any component intended to be removable without the use of a tool shall not wholly fit into a child’s mouth.

Restraint system

The high chair shall be such as to prevent the child from slipping forward out of the seat either by a restraint system comprising a crotch restraint and a horizontal component or by an integral harness. If the high chair is fitted with a reclinable back rest, it shall have an integral harness.

Crotch restraint, waist belts and straps of the integral harness shall have no permanent damage which can impair their safety and normal function when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

When a high chair is fitted with harness or belt attachment points these shall have no permanent damage which can impair their safety and normal function when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

If an integral harness or a belt is supplied it shall be adjustable and it shall have no permanent damage which can impair its safety and normal function when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

Lateral protection

The high chair shall be fitted with arms or other means of lateral protection.

Back rest

The high chair shall be fitted with a back rest of the appropriate height to ensure that a child remains in the high chair when he or she leans in any direction.

Reclinable back rest

The mechanism allowing the back rest of the high chair to be adjusted shall not slip from one position to another when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

Castors and wheels

High chairs may be fitted with a maximum of two wheels or castors.

Structural integrity

The functions of the high chair shall be unimpaired when it is subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair.

Stability

If parts of the high chair are designed to be removable (e.g. a tray or a footrest) the stability requirement applies to the high chair with and without these part(s).

When subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the high chair, the high chair shall not overturn sideways, rearwards or forwards.

## Safety information, user manual and marking

General

The safety information shall be given in the language(s) of the country in which the chair mounted seat is offered for retail sale. All markings shall remain legible and any label used for the marking shall not detach easily.

Safety information

Safety information shall be provided, in a clear and conspicuous way. This information shall have a heading ‘WARNING!’ and shall convey at least the following (or equivalent) messages:

- ‘Never leave the child unattended.’

- ‘Always use the child restraint and system.’

- ‘Always check the security and the stability of the high chair before use.’

The safety information sentences can be replaced by suitable pictograms.

Purchase information

Purchase information shall be available at the point of sale and shall contain at least the following (or equivalent) statement: ‘This product is intended for children up to 3 years old who are able to sit up unaided.’

This information may be given by means of a pictogram.

Marking

High chairs shall be permanently marked with the warning conveying the following (or equivalent) message: ‘WARNING. DO NOT LEAVE THE CHILD UNATTENDED’.

User manual

In the user manual, instructions for use of the high chair shall be provided and shall be headed with the statement ‘IMPORTANT! KEEP FOR FUTURE REFERENCE’ or an equivalent message.

The instructions shall contain the following (or equivalent) warnings headed with ‘WARNING’:

- ‘Do not leave the child unattended’.

- ‘Do not use the high chair unless all components are correctly fitted and adjusted’.

Additionally, the user manual shall provide the following warnings:

- to make sure that any harness is correctly fitted;

- to be aware of the risk of open fire and other sources of strong heat, such as electric bar fires, gas fires etc. in the vicinity of the high chair.

The following safety information shall be provided in the user manual:

- assembly drawing, a list and/or description of all parts and tools required for assembly and a diagram of the bolts and other locking mechanisms required;

- statement not to use the high chair until the child can sit up unaided;

- statement not to use the high chair if any part is broken, torn or missing;

- cleaning and maintenance recommendations, if applicable.

# Table mounted chairs

## Risks

The main risks associated with table mounted chairs are:

- entrapments in gaps and openings;

- falls: if children are not restrained properly or left unrestrained, they may slip out of the chair;

- the table mounted chair can detach from the table and fall down

## Scope

These safety requirements are applicable to table mounted chairs, intended for children who are able to sit by themselves (approximately 6 months old) and weighing up to 15 kg.

## Safety requirements

General

The table mounted chair, when assembled for use, shall be constructed so as to prevent any risk of pinching, cutting or wounding for both the child and the carer.

Entrapment hazards from gaps and openings

To avoid entrapment, table mounted chairs shall not incorporate open ended tubes, or include gaps or openings that that could cause injuries to a child.

The design of table mounted chairs shall prevent the child falling down through the gaps and openings.

Hazardous edges, corners and protruding parts

All accessible edges, corners and protruding parts on the table mounted chair shall be rounded and free from burrs.

Choking hazard

In order to prevent choking hazards, there shall not be any small parts, whether intended to be removed without the use of a tool or not, which could become detached by forces that a child can apply and which could enter entirely into a child’s mouth.

In order to prevent choking hazards, stuffing materials that present choking hazards shall not become accessible when submitted to the forces that a child can apply.

Ingestion hazards

To avoid the hazard of ingestion, separate or small parts which are detachable by forces that a child can apply and which can pass in the oesophagus shall be avoided.

Suffocation hazards

There shall not be any plastic decals which could be gripped by a child and detached by forces that a child can apply. There shall not be any impermeable sheeting which could cover both the mouth and the nose of the child and present suffocation hazards.

Self-tapping screws

Self-tapping screws shall not be used to fasten any component that is designed to be removed or loosened when dismantling the table mounted chair for purposes of transportation or storage.

Hazards due to moving parts

Table mounted chairs designed to fold shall have a folding mechanism incapable of being operated by a child or by inadvertent action made by the carers. It shall not be possible to set up a table mounted chair for normal use without the locking mechanism being activated.

Falling hazards

The table mounted chair shall be fitted with a restraint system that is adjustable to the size of the child and shall be comprised of at least a waist and a crotch restraint. It shall not be possible to use the restraint system without the crotch restraint being used.

The restraint system, straps, anchoring points and fastening system shall not break, become loose or tear away from their support under the internal and external forces capable of being applied by a child.

Dimensions

When mounted for use the height of the backrest of the table mounted chair shall be of the appropriate height and it shall be fitted with sideways armrests of the adequate height as well to ensure that a child remains in the chair when he or she leans in any direction.

Footrests

Footrests shall not be provided.

Removable seats

If the seat is removable from the frame, the fastening devices for the anchoring of the seat to the frame shall be designed so as to prevent the seat from inadvertent detachment from the frame.

The seat shall remain attached to the frame when exposed to mechanical stresses reasonably foreseeable during the normal use of the table mounted chair.

Structural integrity

The table mounted chair shall have no permanent damage which can impair its safety and normal function when subjected to static and dynamic mechanical stresses reasonably foreseeable during the normal use of the table mounted chair.

Anchoring support

Anchoring support shall ensure that the table mounted chair shall not be easily moved when mounted to the support surface. Anchoring of the table mounted chairs shall be resistant to bouncing.

Stability

The table mounted chairs shall not fold or overturn when mechanical stresses to which they are subjected during normal use are applied.

## Safety information, user manual and marking

General

The safety information shall be marked on the product or included in the instructions to the user.

The safety information shall be given in the language(s) of the country in which the table mounted chair is offered for retail sale. All markings shall remain legible and any label used for the marking shall not detach easily.

Safety information

Safety information shall be provided, in a clear and conspicuous way. The table mounted chair shall have the safety information visible after attachment to the table and before the child is put into the table mounted chair. This information shall have a heading ‘WARNING!’ and shall convey at least the following (or equivalent) messages:

- ‘Never leave the child unattended.’

- ‘Always use the child restraint and chair attachment systems.’

- ‘Always check the security and the stability of the table mounted chair before use.’

- ‘This table mounted chair is not suitable for use on all tables. Do not use on glass topped tables, tables with loose table tops, table leaves, single pedestal tables, card tables or camping tables’.

The safety information sentences can be replaced by suitable pictograms.

Purchase information

Purchase information shall be available at the point of sale and shall contain at least:

- the following (or equivalent) statement: ‘This product is intended for children up to 3 years old, weighing a maximum of 15 kg, who are able to sit up unaided.’;

- the adequate dimensions of the supporting surface to which the table mounted chair can be attached;

This information may be given by means of a pictogram.

Marking

The table mounted chair shall be visibly and permanently marked with at least the following (or equivalent):

- the warning ‘WARNING ! Never leave the child unattended’ shall be visible when the table mounted chair is in position of use;

- the statement ‘Maximum weight: 15 kg’.

User manual

Instructions concerning correct and safe assembly and use of the table mounted chair shall be provided in the user manual. These instructions shall be precise, unambiguous and shall include at least the following (or equivalent) messages:

- ‘Read the instructions carefully before use and keep them for future reference. Your child may be hurt if you do not follow the instructions’.

- ‘The table mounted chair is not suitable for children who cannot sit unaided’.

- ‘WARNING! Never leave the child unattended.

- ‘Check that the table will not tip over when the table mounted chair, attached to it, is in use’.

- ‘Do not use tablecloths or other objects on the support surface which might interfere with the proper functioning of the anchoring elements. Keep the table structure and surface clean and dry’.

- ‘The table mounted chair shall not be used by children weighing more than 15 kg’.

- ‘Routinely check any clamping screws and retighten them, if required.’

- ‘WARNING: Do not use the table mounted chair if any components are broken or missing’.

- ‘Do not use replacement parts other than those approved by the manufacturer or distributor’.

- ‘Do not attach the table mounted chair where the child might use its feet to push against a part of the table, another chair or any other structure as this could cause the table mounted chair to come off the table’.

- ‘Do not use on glass topped tables, tables with loose table tops, table leaves, single pedestal tables, card tables or camping tables’.

The adequate dimensions of support surface shall be specified.

OJ L 11, 15.1.2002, p. 4.

OJ L 24, 21.7.1998, p. 37.

OJ C 91, 16.04.2003, p. 7.

as defined in the Decision No 768/2008/EC of the European Parliament and of the Council of 9 July 2008 on a common framework for the marketing of products, and repealing Council Decision 93/465/EEC